# WiseDrive OBD2 SDK

# Client Integration Guide

Version 2.0.0  |  April 2026

Confidential - For Authorized Clients Only

# Table of Contents

1. Overview

2. Requirements

3. Step 1: Add Repository

4. Step 2: Add SDK Dependency

5. Step 3: Add Permissions

6. Step 4: Initialize SDK

7. Step 5: Scan for OBD Devices

8. Step 6: Connect & Run Scan

9. Step 7: Receive Data on Your Backend

10. JSON Payload Reference

11. Live Data PIDs Reference

12. Testing with Mock Mode

13. Error Handling

14. FAQ

15. Support

# 1. Overview

The WiseDrive OBD2 SDK enables your Android application to perform comprehensive vehicle diagnostics using Bluetooth-connected ELM327 OBD-II adapters. The SDK handles all the complexity of Bluetooth communication, OBD-II protocol handling, and data extraction.

Key capabilities:

Automatic Bluetooth device discovery (Classic SPP + BLE)

5-strategy connection fallback for maximum compatibility

Full DTC scanning across all vehicle ECU modules (23+ manufacturers)

19 live vehicle data parameters (RPM, speed, temperature, etc.)

Scan data delivered to your backend as plain JSON via HTTP POST

Built-in error handling and automatic retries

# 2. Requirements

| Requirement | Version |
| --- | --- |
| Android | API 21+ (Android 5.0 Lollipop) |
| Kotlin | 1.9+ |
| Gradle | 8.0+ |
| AGP | 8.2+ |
| Bluetooth | Classic SPP or BLE |
| Hardware | ELM327-compatible OBD-II adapter |

# 3. Step 1: Add Repository

Add the WiseDrive private Maven repository to your project.

File: settings.gradle.kts

dependencyResolutionManagement {
    repositories {
        google()
        mavenCentral()
        
        // WiseDrive Private Repository
        maven {
            url = uri("https://wisedrive.jfrog.io/artifactory/wisedrive-sdk-releases")
            credentials {
                username = "YOUR_CLIENT_USERNAME"  // Provided by WiseDrive
                password = "YOUR_CLIENT_TOKEN"     // Provided by WiseDrive
            }
        }
    }
}

Note: Contact sdk@wisedrive.in to receive your client credentials.

# 4. Step 2: Add SDK Dependency

File: app/build.gradle.kts

dependencies {
    implementation("com.wisedrive:obd2-sdk:2.0.0")
}

Sync your project after adding the dependency.

# 5. Step 3: Add Permissions

File: AndroidManifest.xml

<!-- Bluetooth permissions -->
<uses-permission android:name="android.permission.BLUETOOTH" />
<uses-permission android:name="android.permission.BLUETOOTH_ADMIN" />
<uses-permission android:name="android.permission.BLUETOOTH_CONNECT" />
<uses-permission android:name="android.permission.BLUETOOTH_SCAN" />
<uses-permission android:name="android.permission.ACCESS_FINE_LOCATION" />

<!-- Internet for data submission -->
<uses-permission android:name="android.permission.INTERNET" />

# 6. Step 4: Initialize SDK

import com.wisedrive.obd2.WiseDriveOBD2SDK
import com.wisedrive.obd2.models.SDKConfig

class MyActivity : AppCompatActivity() {

    private lateinit var sdk: WiseDriveOBD2SDK

    override fun onCreate(savedInstanceState: Bundle?) {
        super.onCreate(savedInstanceState)
        
        val config = SDKConfig(
            apiKey = "your-api-key",
            useMock = false,
            clientEndpoint = "https://your-server.com/api/obd-data",
            licensePlate = "MH12AB1234"
        )
        
        sdk = WiseDriveOBD2SDK.initialize(this, config)
    }
}

## Configuration Parameters

| Parameter | Type | Required | Description |
| --- | --- | --- | --- |
| apiKey | String | Yes | Your WiseDrive API key |
| useMock | Boolean | No | Use mock data for testing (default: false) |
| clientEndpoint | String | No | Your backend URL to receive scan data |
| licensePlate | String | Yes | Vehicle registration/license plate number |

# 7. Step 5: Scan for OBD Devices

// Start scanning for Bluetooth OBD devices
sdk.startDeviceScan { device ->
    println("Found: ${device.name} (${device.id})")
    addDeviceToList(device)
}

// Stop scanning when device is selected
sdk.stopDeviceScan()

# 8. Step 6: Connect & Run Scan

// Connect to selected OBD device
sdk.connect(deviceId = selectedDevice.id)

// Run full diagnostic scan
sdk.runFullScan(
    licensePlate = "MH12AB1234",
    trackingId = "ORDER-12345"
) { progress ->
    println("Phase: ${progress.phase}, Status: ${progress.status}")
    updateProgressUI(progress)
}

# 9. Step 7: Receive Data on Your Backend

The SDK automatically sends scan results to your clientEndpoint as plain JSON via HTTP POST. No decryption or special handling is needed on your side.

## Python (FastAPI) Example

from fastapi import FastAPI, Request

app = FastAPI()

@app.post("/api/obd-data")
async def receive_obd_data(request: Request):
    scan_data = await request.json()
    
    license_plate = scan_data["inspectionId"]
    dtcs = scan_data["diagnosticTroubleCodes"]
    live_data = scan_data["liveData"]
    
    for dtc in dtcs:
        print(f"  {dtc['code']}: {dtc['description']}")
    
    return {"status": "received", "dtcCount": len(dtcs)}

## Node.js (Express) Example

const express = require('express');
const app = express();
app.use(express.json({ limit: '10mb' }));

app.post('/api/obd-data', (req, res) => {
    const scanData = req.body;
    
    console.log(`Vehicle: ${scanData.inspectionId}`);
    console.log(`DTCs: ${scanData.summary.totalDTCs}`);
    
    scanData.diagnosticTroubleCodes.forEach(dtc => {
        console.log(`  ${dtc.code}: ${dtc.description}`);
    });
    
    res.json({ status: 'received' });
});

app.listen(3000);

# 10. JSON Payload Reference

Your backend receives the following JSON structure:

{
  "inspectionId": "MH12AB1234",
  "scanId": "scan_1234567890",
  "scanTimestamp": "2026-04-15T10:30:00Z",
  "scanDuration": 45000,
  "vehicle": {
    "vin": "KMHXX00XXXX000000",
    "manufacturer": "Hyundai",
    "protocol": "ISO 15765-4 CAN (11 bit ID, 500 kbps)"
  },
  "milStatus": { "on": true, "dtcCount": 3 },
  "diagnosticTroubleCodes": [
    {
      "code": "P0300",
      "description": "Random/Multiple Cylinder Misfire",
      "severity": "Critical",
      "possibleCauses": ["Faulty spark plugs", ...],
      "symptoms": ["Engine misfiring", ...],
      "solutions": ["Replace spark plugs", ...],
      "ecuSource": "Engine/PCM"
    }
  ],
  "liveData": [
    { "pid": "0C", "name": "Engine RPM", 
      "value": 850.0, "unit": "RPM" }
  ],
  "summary": {
    "totalDTCs": 3,
    "byType": { "critical": 1, "important": 1, "nonCritical": 1 },
    "totalLiveReadings": 19
  }
}

# 11. Live Data PIDs Reference

| PID | Name | Unit | Range |
| --- | --- | --- | --- |
| 0C | Engine RPM | RPM | 0 - 16,383 |
| 0D | Vehicle Speed | km/h | 0 - 255 |
| 05 | Coolant Temperature | C | -40 to 215 |
| 11 | Throttle Position | % | 0 - 100 |
| 04 | Engine Load | % | 0 - 100 |
| 0F | Intake Air Temperature | C | -40 to 215 |
| 0B | Intake Manifold Pressure | kPa | 0 - 255 |
| 2F | Fuel Tank Level | % | 0 - 100 |
| 42 | Control Module Voltage | V | 0 - 65.5 |
| 46 | Ambient Air Temperature | C | -40 to 215 |
| 10 | MAF Air Flow Rate | g/s | 0 - 655 |
| 0E | Timing Advance | deg | -64 to 63.5 |
| 0A | Fuel Pressure | kPa | 0 - 765 |
| 5C | Engine Oil Temperature | C | -40 to 210 |
| 5E | Engine Fuel Rate | L/h | 0 - 3,276 |
| 1F | Run Time Since Start | sec | 0 - 65,535 |
| 21 | Distance with MIL On | km | 0 - 65,535 |
| 06 | Short Term Fuel Trim B1 | % | -100 to 99.2 |
| 07 | Long Term Fuel Trim B1 | % | -100 to 99.2 |

# 12. Testing with Mock Mode

For development without a physical OBD device:

val config = SDKConfig(
    apiKey = "your-api-key",
    useMock = true,  // Generates realistic mock data
    clientEndpoint = "https://your-server.com/api/obd-data",
    licensePlate = "TEST1234"
)

val sdk = WiseDriveOBD2SDK.initialize(this, config)
sdk.runFullScan(licensePlate = "TEST1234", trackingId = "TEST-001") { 
    progress -> println("${progress.phase}: ${progress.status}")
}

# 13. Error Handling

try {
    sdk.connect(deviceId = device.id)
    sdk.runFullScan(licensePlate = plate, trackingId = orderId) { 
        progress -> // Handle progress
    }
} catch (e: BluetoothNotAvailableException) {
    // Bluetooth not enabled
} catch (e: DeviceConnectionException) {
    // Failed to connect to OBD device  
} catch (e: ScanTimeoutException) {
    // Scan timed out
} catch (e: Exception) {
    // General error
}

# 14. FAQ

Q: Do I need to decrypt the data?

A: No. Your backend receives plain JSON. Only WiseDrive internal analytics receives encrypted data.

Q: What if the OBD device disconnects?

A: The SDK has automatic retry logic and will report partial results if the connection is lost.

Q: Can I filter which data fields I receive?

A: The SDK sends the full scan report. Filter the fields you need on your backend.

Q: What Bluetooth adapters are supported?

A: Any ELM327-compatible adapter. Popular: VEEPEAK, BAFX, Konnwei, Carista, BlueDriver.

Q: What vehicles are supported?

A: All OBD-II compliant vehicles (1996+ in US, 2001+ in EU, 2005+ in India). Optimized for 23+ manufacturers.

# 15. Support

Email: sdk@wisedrive.in

Documentation: https://wisedrive.in/sdk/docs

SDK Version: 2.0.0